Veriton 4000 Compact Tower

Acer Veriton Vero 6000 Mini

Veriton 4000 Compact Tower

SPECIFICATIONS	Veriton 	VN6735GT

8

9

10

11

1222

7

15

1333

14

16

1

2

3

4

6

5

| Product views | - One button recovery - Universal audio jack - USB 3.2 Gen2x2 (20 Gbps) Type-CTM port - USB 3.2 Gen 2 10 Gbps Type-A port - USB 3.2 Gen 2 10 Gbps Type-A port | - Power button - External Antenna (optional) - Kensington lock slot - DisplayPort™ - DisplayPort™ - HDMI® port - 2x USB 3.2 Gen2 5Gbps Type-A - 2x USB 3.2 Gen1 10Gbps Type-A | - Ethernet RJ45 LAN port - Power connector (DC-in) - HDMI® port |  |
| --- | --- | --- | --- | --- |
| Operating system1, 2 | Windows 11 Pro 64-bit (Acer recommends Windows 11 Pro for business.) Windows 11 Home 64-bit UEFI Shell |  |  |  |
| CPU and chipset1 | •Intel® Core™ Ultra 9 processor 285T, Intel® Graphics • Intel® Core™ Ultra 7 processor 265T, Intel® Graphics • Intel® Core™ Ultra 5 processor 245T, Intel® Graphics • Intel® Core™ Ultra 5 processor 235T, Intel® Graphics •Intel® Core™ Ultra 5 processor 225T, Intel® Graphics Chipset: Intel® Q870 Chipset • Intel® Q870 Chipset |  | • Intel® Core™ Ultra 9 processor 285, Intel® Graphics • Intel® Core™ Ultra 7 processor 265, Intel® Graphics • Intel® Core™ Ultra 5 processor 245, Intel® Graphics •Intel® Core™ Ultra 5 processor 235, Intel® Graphics • Intel® Core™ Ultra 5 processor 225, Intel® Graphics |  |
| Memory1, 3, 4 | 32 GB DDR5 5600 MT/S soDIMM 16 GB DDR5 5600 MT/S soDIMM 8 GB DDR5 5600 MT/S soDIMM Up to 64 GB of Dual-channel DDR5 5600 MT/S |  |  |  |
| Storage1, 5 | Hard disk drive: - 2048 GB / 1024 GB / 512 GB M.2 2280 PCI-e Gen4 Performance SSD - 1024 GB / 512 GB / 256 GB M.2 2280 PCI-e Gen4 SSD |  |  |  |
| Audio | Integrated high-definition sound |  |  |  |
| Communication | WLAN: 802.11be/ax/ac/a/b/g/n, Wi-Fi 7 and Bluetooth® 5.4 802.11ax/ac/a/b/g/n, Wi-Fi 6E and Bluetooth® 5.3 |  | LAN: Gigabit Ethernet |  |
| Expansion slot(s) | M.2 slot (for SSD): 3 M.2 slot (for WLAN): 1 |  |  |  |
| Commercial features6,7 | VVN4735G(T) M.2 slot (for WLAN): 1 TPM v2.0 (Trusted Platform Module) Intrusion alert (via BIOS) Hard drive password protection (via BIOS) One-button recovery (OBR) WOL (Wake On Lan) PXE (Preboot eXecution Environment) Kensington lock slot Intel® Active Management Technology Intel® vPro™ Technology RAID support (via BIOS) |  |  |  |
| Commercial selected options8 | Additional D-SUB port Additional 2nd HDMI® port Additional 2.5G LAN port Additional USB Type-A 3.2 Gen 2 port USB 3.2 Gen 2 Type-C port supporting data transfer, 5V/3A USB charger, video output and 100W PD-in USB 3.2 Gen 2 Type-C port supporting data transfer, 5V/3A USB charger, and video output Internal speaker COM (Series) port |  |  |  |
| Dimensions and weight | Dimensions: • 186.3 (W) x 186.3 (D) x 36.2 (H) mm (7.33 x 7.33 x 1.43 inches) |  | Weight: • 1.3 kg (2.87 lbs.) |  |
| Power supply and battery | 3-pin 135 W AC adapter 3-pin 180 W AC adapter |  |  |  |
| System compliance9 | RoHS, DMI, GS, ENERGY STAR®, PC2001, EPEAT Gold ® design (EPEAT registration varies by country) |  |  |  |
| Options and accessories | USB keyboard and mouse VESA mount kit (100mm*100mm) (optional) |  |  |  |
| Certification | BSMI VCCI CE FCC |  | CB EPEAT Gold |  |
| - Specifications vary depending on model. - 64-bit software is required to enjoy the advantages of 64-bit processing. - A 64-bit operating system is required to enjoy the ultimate performance of 4 GB or higher memory. - Memory speed may vary, depending on the CPU, chipset or memory fitted. - 1 GB is 1 billion bytes. Actual formatted capacity is less and may vary depending on preloaded materials and operating environment. Recovery Management uses a portion of the stated hard disk capacity as dedicated backup space. - Intel® AMT is a feature of Intel® Core™ processors with Intel® vPro™ Technology - Intel® vPro™ technology is sophisticated and requires setup and activation. Availability of features and results will depend upon the setup and configuration of your hardware, software, and IT environment. To learn more visit Intel website. - Only one of the additional options can be selected. - EPEAT® registration varies by country and configuration. See http://www.epeat.net/ for registration status for each country. |  |  |  |  |