Swift Edge 16 AI

SPECIFICATIONS 	              Swift Edge 16 AI	SFE16-I51 | SFE16-I5T

Acer NITRO V 16S                                                               ANV16S-71

| Operating System |  | Windows 11 Home |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CPU and Chipset |  | Intel® Core™ Ultra 9 processor 386H |  |  |  |  |  |  |
| Memory |  | Dual-channel LPDDR5X SDRAM support 16/24/32 GB of onboard LPDDR5X system memory | Memory Frequency : Up to 7467 MT/s |  |  |  |  |  |
| Display |  | Swift SFE16-I51 16.0" display with OLED (Organic Light-Emitting Diode) technology ¦ WQXGA+ 2880 x 1800 ¦ 400 nits brightness, typical ¦ Acer CineCrystal™ ¦ 120 Hz refresh rate ¦ VESA DisplayHDR™ True Black 500 certified 16:10 aspect ratio 100% DCI-P3 color gamut, typical Peak brightness up to 500 nits in HDR Wide viewing angle up to 170 degrees Mercury free, environment friendly |  | Swift SFE16-I51 16.0" display with OLED (Organic Light-Emitting Diode) technology ¦ WUXGA 1920 x 1200 ¦ 300 nits brightness, typical ¦ Acer CineCrystal™ 16:10 aspect ratio 100% DCI-P3 color gamut, typical Wide viewing angle up to 170 degrees Mercury free, environment friendly |  |  |  | Swift SFE16-I51T 16.0" display with OLED (Organic Light-Emitting Diode) technology ¦ WQXGA+ 2880 x 1800 ¦ 340 nits brightness ¦ Acer CineCrystal™ Integrated multi-touch 16:10 aspect ratio 100% DCI-P3 color gamut Wide viewing angle up to 170 degrees Mercury free, environment friendly |
| Graphics |  | Intel® Graphics eligible Hardware-accelerated video decode ¦ AVC ¦ HEVC ¦ VP9 ¦ AV1 ¦ VVC Hardware-accelerated video encode ¦ AVC ¦ HEVC ¦ AV1 Microsoft DirectX® 12 OpenGL® 4.6 OpenCL™ 3.0 |  |  |  |  |  |  |
| Audio |  | DTS® X:Ultra Audio, featuring optimized Bass, Loudness, Speaker Protection with up to 6 custom content modes via embedded smart amplifier Immersive audio rendering over headphones and internal speakers Acer Ultra Boost Stereo Speaker Module & PCR 50% material chamber design High SNR DAC with 2.1 Vrms out capability and can driver high impedance headphone (up to 600 ohm) Acer Purified.Voice technology with AI noise reduction in 3 mics array built-in microphones to support Advanced Privacy Mode.: Features include far-field pickup, dynamic noise reduction through neural network, adaptive beam forming, and pre-defined personal and conference call modes. |  |  |  |  | Compatible with Cortana with Voice Acer TrueHarmony Technology for lower distortion, wider frequency range, headphone-like audio and powerful sound Soundwire Codec with MSFT SDCA 1.0 driver supported Hi-Res Audio Certification by JAS ( Headphone Output setting supported up to 24bit /192KHz) |  |
| Storage |  | Solid State Drive: 512 GB / 1 TB PCIe Gen4, 16 Gb/s, NVMe |  |  |  |  |  |  |
| Webcam |  | Video Conferencing UFC with narrow USB FHD camera and IR camera ¦ Blue glass lens ¦ 1920 x 1080 resolution ¦ 1080p video with temporal noise reduction ¦ Staggered high dynamic range (SHDR) ¦ Dual microphones 33 mm + 33 mm ¦ 88 x 3.2 x 2.6 mm ¦ Windows compatible only |  |  |  |  |  |  |
| Wireless and Networking |  | WLAN: Killer™ Wi-Fi 7 Wireless LAN 802.11 a/b/g/n/ac/ax/be Band: 2.4 GHz, 5 GHz, and 6 GHz 2x2 MU-MIMO technology Multi-Link Operation | WPAN: Supports Bluetooth® 5.4 or above |  |  |  |  |  |
| Security |  | MSFT Pluton Security Processor as Firmware TPM solution ¦ Camera shutter |  |  |  |  |  |  |
| Dimensions and Weight |  | Dimensions 357.15 (W) x 244.64 (D) x 7.6/14.6 (H) mm (14.06 x 9.63 x 0.3/0.57 inches) with Metal A Cover |  |  | Weight 1.25 kg with 3-cell battery pack, WQXGA display (SFE16-I5) 1.31 kg with 3-cell battery pack, WQXGA display SFE16-I5T |  |  |  |
| Power Adapter and Battery |  | Power Adapter 3-pin 100 W PD PCR 50%, TCO 9.0 AC adapter 5 V / 3 A or 9 V / 3 A or 12 V / 3 A or 15 V / 3 A or 20 V / 3.25 A as output power 108.0 (W) x 56.0 (D) x 26.5 (H) mm (4.25 x 2.20 x 1.04 inches) 296 g ± 5% with 150 cm DC cable Black color with Acer logo Brick type AC pins | Battery 65 Wh 5614 mAh 11.58 V 3-cell Li-ion battery pack For models with WQXGA Plus Panel + SSD Battery life: Up to 12.5 hours (based on MobileMark® 2030 test results) Up to 20 hours (based on video playback test results) Up to 13.5 hours (based on web browsing test results) |  |  |  |  |  |
| Input and Control |  | Keyboard 99 / 100 / 103 keys ¦ Acer backlit keyboard layout ¦ Numeric keypad ¦ International language support Slim design ¦ 1.3 mm key travel Dedicated AcerSense key Programmable key ¦ MyKey Enlarged keycaps Adjustable backlight brightness Quiet design for noise reduction Caps Lock indicator Microphone mute indicator Function Lock key Numeric keypad |  | Touchpad Multi-gesture touchpad ¦ Two-finger scroll ¦ Pinch gesture ¦ Gestures to open Cortana ¦ Action Center ¦ Multitasking ¦ Application command gestures Microsoft Precision Touchpad certification Moisture resistant Corning® Gorilla® Glass |  |  |  | Control Keys Programmable key ¦ My Key Microsoft Copilot key AcerSense key |
| Windows Desktop Apps |  | In-House Applications Acer AI Image Generator ¦ Acer LiveArt ¦ Acer ORD Service ¦ Acer QuickPanel ¦ Acer VisionArt |  | Search AcerSense |  | Tools Acer My Key Application |  |  |
| Warranty |  | One-year International Travelers Warranty (ITW) |  |  |  |  |  |  |
| Disclaimers |  |  |  |  |  |  |  |  |
| • | Specifications vary depending on model. |  |  |  |  |  |  |  |
| • | Shared system memory may be allocated to support graphics, depending on system memory size and other factors. Actual system memory available to the operating system will be reduced by any memory used by the graphics solution and resources required by the operating environment. |  |  |  |  |  |  |  |
| • | Memory speed may vary, depending on the CPU, chipset or memory fitted. |  |  |  |  |  |  |  |
| • | All brands and product names mentioned herein include trademarks of their respective companies and are used solely to describe or identify the products. |  |  |  |  |  |  |  |
| • | Not all features indicated are supported on all SKUs |  |  |  |  |  |  |  |
| • | 1 GB is 1 billion bytes. Actual formatted capacity is less and may vary depending on preloaded materials and operating environment. Acer Recovery Management uses a portion of the stated hard disk capacity as dedicated backup space. |  |  |  |  |  |  |  |
| • | Minor variations in system dimensions are possible due to the manufacturing process. |  |  |  |  |  |  |  |
| • | Weight may vary depending on product configuration, vendor components, manufacturing variability, and selected options. |  |  |  |  |  |  |  |
| • | Listed battery life is based on testing using video playback. The test clip loops and continues to run, without interruption or any other inputs, until the battery is drained. Stated battery life is for comparison purpose only. Actual battery life varies by model, configuration, applications, video chip format and features used. A battery's maximum capacity decreases with time and use. Battery life test configuration: Intel® ULTRA 7 (Series 3), Intel® Graphics, WQXGA+ OLED, SSD only, 32GB of LPDDR5X memory. Battery life test condition: LCD brightness 150 nits, connected headphones, WLAN off, Bluetooth off and Better Battery Mode on. |  |  |  |  |  |  |  |
| • | Listed battery life is based on testing using Web browsing. The test clip loops the loading of 50 selected websites using Microsoft Edge every 30 seconds, until the battery is drained. Stated battery life is for comparison purpose only. Actual battery life varies by model, configuration, applications, video chip format and features used. A battery's maximum capacity decreases with time and use. Battery life test configuration: Intel® ULTRA 7 (Series 3), Intel® Graphics, WQXGA+ OLED, SSD only, 32GB of LPDDR5X memory. Battery life test condition: 100 nits LCD brightness, 50% speaker volume, WLAN on, Bluetooth on, no WW direct Internet connection. |  |  |  |  |  |  |  |
| • | Actual battery life may vary considerably by specifications, depending on product model, configuration, applications, power management settings, operating conditions, and features utilized. Performance variation also arises based on components in use, which includes but is not limited to the processor, RAM capacity, storage, display and resolution, etc. |  |  |  |  |  |  |  |
| • | Bundled software may vary depending on hardware configuration, OS and regional availability. |  |  |  |  |  |  |  |

Product Sheet is based on Swift SFE16-I51T (Win11) Specifications (v1-0-0) (Preview) and Swift SFE16-I51 (Win11) Specifications (v1-0-0) (Preview)

In a continuing effort to improve the quality of our products, information in this document is subject to change without notice. Images shown are only representations of some of the configurations available for this model. Availability may vary depending on region. As a phenomenon known to thin-film transistors (TFTs), liquid crystal displays (LCDs) commonly exhibit a small number of discoloration dots, as so-called "non-conforming pixels." This phenomenon is a limitation of TFT LCD technology, not a product defect, and as such is not covered by Acer's warranty.
© 2026. All rights reserved.

Microsoft and Windows are registered trademarks of Microsoft Corporation.
3D XPoint, Agilex, Altera, Arc, Arria, Avalon, Axxia, Barefoot Networks, the Barefoot logo, the Footsie logo, Celeron, Cyclone, Docae, DoubleShot, eASIC, easicopy, FlexRAN, Gaudi, Granulate, HyperFlex, Intel, the Intel logo, Intel Adapix, Intel Agilex, Intel Atom, Intel Cofluent, Intel Core, Intel Evo, Intel Geti, Intel Inside, the Intel Inside logo, Intel Optane, Intel RealSense, Intel SpeedStep, Intel Tiber, Intel Unison, Intel vPro, Iris, Killer, Linux, MAX, Movidius, Myriad, Nios, OmniBridge, OpenVINO, the OpenVINO logo, Pentium, Quartus, Simics, SmartByte, SoFiaSilicon, Sound Mark, StarPro, Stay With It, the Engineering Stay With It logo, Stratix, the Stratix logo, the Journey Inside, Thunderbolt, the Thunderbolt logo, Tofino, Ultrabook, VTune, and Xeon are trademarks of Intel Corporation or its subsidiaries.

About Acer

Founded in 1976, Acer is one of the world's top ICT companies with a presence in more than 160 countries. As Acer evolves with the industry and changing lifestyles, it is focused on enabling a world where hardware, software and services will fuse with one another, creating ecosystems and opening up new possibilities for consumers and businesses alike. Acer's 7,500 employees are dedicated to the research, design, marketing, sale, and support of products and solutions that break barriers between people and technology. Please visit www.acer.com for more information.