TravelMate P4 Spin 14 AI

| SPECIFICATIONS |  |  |  |  | Acer TravelMate P4 Spin 14 AI |  |  | P414RN-56 ¦ P414RN-76 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Operating System |  | Windows 11 Pro (Acer recommends Windows 11 Pro for business.) Windows 11 Home |  |  |  |  |  |  |
| CPU and Chipset |  | P414RN-56 Intel® Core™ Ultra 5 processor 322 Intel® Core™ Ultra 5 processor 325 |  |  | Intel® Core™ Ultra 5 processor 335 Intel® Core™ Ultra 5 processor 355 Intel® Core™ Ultra 5 processor 365 |  |  | P414RN-76 Intel® Core™ Ultra 7 processor 356H |
| Memory |  | Dual-channel DDR5 SDRAM ¦ Two SO-DIMM modules ¦ Up to 64 GB system memory ¦ Up to 5600 MT/s memory frequency |  |  |  |  |  |  |
| Display |  | 14.0" display with IPS (In-Plane Switching) technology ¦ WQXGA+ 2880 x 1800 ¦ Integrated multi-touch 16:10 aspect ratio ¦ 100% sRGB color gamut ¦ Wide viewing angle ¦ Mercury free, Environment friendly |  |  |  | 14.0" display with IPS (In-Plane Switching) technology ¦ WUXGA 1920 x 1200 ¦ LED-backlit LCD ¦ Integrated multi-touch 16:10 aspect ratio ¦ Super-slim design ¦ Mercury free, Environment friendly ¦ AES 2.0 stylus support |  |  |
| Graphics |  | Intel® Graphics eligible Graphics Special Technology: AVC, VP9, AV1 HW Accelerated Video Decode ¦ AVC, AV1 HW Accelerated Video Encode ¦ Microsoft DirectX 12 ¦ OpenGL 4.6 ¦ OpenCL 3.0 |  |  |  |  |  |  |
| Audio |  | DTS® X:Ultra Audio, featuring optimized Bass, Loudness, Speaker Protection with up to 3 custom content modes via smart amplifier Smart Auto switch to Voice enhanced Mode via VOIP High SNR DAC with 2.1 Vrms out capability and can driver high impedance headphone (up to 600 ohm) Compatible with Cortana with Voice |  |  |  | Acer Purified.Voice technology with AI noise reduction in 3 mics array built-in microphones to support Advanced Privacy Mode. Features include far-field pickup, dynamic noise reduction through neural network, adaptive beam forming, and pre-defined personal and conference call modes. Acer TrueHarmony Technology for lower distortion, wider frequency range, headphone-like audio and powerful sound |  |  |
| Storage |  | Solid state drive: 1 x SSD slot ¦ 256 GB / 512 GB / 1 TB ¦ PCIe Gen 4 ¦ 16 Gb/s ¦ NVMe |  |  |  |  |  |  |
| Webcam |  | Video Conferencing UFC with double T-type USB 5 MP camera ¦ 2880 x 1800 resolution ¦ 1800p 5 MP video at 30 fps with temporal noise reduction ¦ Dual microphones (33 mm + 33 mm) ¦ Blue glass lens ¦ Windows compatible only UFC with USB 5 MP and IR camera ¦ 2880 x 1800 resolution ¦ 1800p 5 MP video at 30 fps with temporal noise reduction ¦ Dual microphones ¦ Blue glass lens ¦ Windows compatible only |  |  |  |  |  |  |
| Wireless and Networking |  | WLAN: Intel® Wi-Fi 7 wireless LAN ¦ 802.11 a/b/g/n/ac/ax/be ¦ 2x2 MIMO technology ¦ 2.4 GHz ¦ 5 GHz ¦ 6 GHz ¦ Intel vPro® Enterprise ¦ Intel® Connectivity Performance Suite (ICPS) |  |  |  | WPAN: Bluetooth® 5.4 or above ¦ Bluetooth LE Audio (LC3 LAN: Gigabit Ethernet, Wake-on-LAN ready) |  |  |
| Security |  | Power keycap fingerprint reader ¦ On-chip matching design Acer ProShield Plus security manager includes: Data protection: File and folder encryption and decryption, Personal Security Drive ¦ Data removal: File shredding ¦ Security alert: Invalid access alert, Security report Discrete Trusted Platform Module (TPM) solution ¦ BIOS passwords: User, Supervisor, HDD (OPAL) ¦ Kensington lock (Nano-type) ¦ Camera shutter |  |  |  |  |  |  |
| Dimensions and Weight |  | Dimensions 312.8 (W) x 231.6 (D) x 16.9/17.1 (H) mm (12.31 x 9.12 x 0.67/0.67 inches) with Metal A Cover |  |  |  | Weight 1.365 kg (3.01 lbs.) with 4-cell battery pack, WQXGA display |  |  |
| Power Adapter and Battery |  | Power Adapter |  |  |  | Battery |  |  |
| Input and Control |  | Keyboard 80 / 81 / 84 keys ¦ Acer backlit keyboard layout ¦ International language support Caps Lock indicator ¦ F4 microphone mute indicator Anti-bacterial treatment (QV03P_AM10BWL) Spill-resistant keyboard ¦ MIL-STD-810H Method 506.6 Procedure III drip test |  |  |  | Touchpad Multi-gesture touchpad ¦ Two-finger scroll ¦ Pinch gesture ¦ Gestures to open Cortana ¦ Action Center ¦ Multitasking ¦ Application commands Microsoft Precision Touchpad certification ¦ Moisture resistant ¦ Corning® Gorilla® Glass Microsoft Precision Touchpad certification ¦ Moisture resistant Control keys Microsoft Copilot key ¦ TravelMate Sense key |  |  |
| Windows Desktop Apps |  | In-House Applications Acer LiveArt ¦ AcerSense | Tools Acer My Key Application |  |  |  |  |  |
| Warranty |  | One-year International Travelers Warranty (ITW) |  |  |  |  |  |  |
| Disclaimers |  |  |  |  |  |  |  |  |
| • | Specifications vary depending on model. |  |  |  |  |  |  |  |
| • | Shared system memory may be allocated to support graphics, depending on system memory size and other factors. Actual system memory available to the operating system will be reduced by any memory used by the graphics solution and resources required by the operating environment. |  |  |  |  |  |  |  |
| • | Memory speed may vary, depending on the CPU, chipset or memory fitted. |  |  |  |  |  |  |  |
| • | All brands and product names mentioned herein include trademarks of their respective companies and are used solely to describe or identify the products. |  |  |  |  |  |  |  |
| • | Not all features indicated are supported on all SKUs |  |  |  |  |  |  |  |
| • | 1 GB is 1 billion bytes. Actual formatted capacity is less and may vary depending on preloaded materials and operating environment. Acer Recovery Management uses a portion of the stated hard disk capacity as dedicated backup space. |  |  |  |  |  |  |  |
| • | Minor variations in system dimensions are possible due to the manufacturing process. |  |  |  |  |  |  |  |
| • | Weight may vary depending on product configuration, vendor components, manufacturing variability, and selected options. |  |  |  |  |  |  |  |
| • | Bundled software may vary depending on hardware configuration, OS and regional availability. |  |  |  |  |  |  |  |
| • | EPEAT® registration varies by country. See http://www.epeat.net/ for registration status for each country. |  |  |  |  |  |  |  |

Specifications based on TravelMate P414RN-56 Specifications (v1-0-0) (Preview) and TravelMate Spin P414RN-76 Specifications

In a continuing effort to improve the quality of our products, information in this document is subject to change without notice. Images shown are only representations of some of the configurations available for this model. Availability may vary depending on region.

As a phenomenon known to thin-film transistors (TFTs), liquid crystal displays (LCDs) commonly exhibit a small number of discoloration dots, as so-called "non-conforming pixels." This phenomenon is a limitation of TFT LCD technology, not a product defect, and as such is not covered by Acer's warranty.

© 2026. All rights reserved.

Microsoft and Windows are registered trademarks of Microsoft Corporation.

3D XPoint, Agilex, Altera, Arc, Arria, Avalon, Axxia, Barefoot Networks, the Barefoot logo, the Footsie logo, Celeron, Cyclone, Docea, DoubleShot, eASIC, easicopy, FlexRAN, Gaudi, Granulate, Hyperflex, Intel, the Intel logo, Intel Adaptix, Intel Agilex, Intel Atom, Intel CoFluent, Intel Core, Intel Evo, Intel Geti, Intel Inside, the Intel Inside logo, Intel Optane, Intel RealSense, Intel SpeedStep, Intel Tiber, Intel Unison, Intel vPro, Iris, Killer, Linutronix, MAX, Movidius, Myriad, Nios, OmniBridge, OpenVINO, the OpenVINO logo, Pentium, Quartus, Simics, SmartByte, SoftSilicon, Sound Mark, StarPro, Stay With It, the Engineering Stay With It logo, Stratix, the Stratix logo, The Journey Inside, Thunderbolt, the Thunderbolt logo, Tofino, Ultrabook, VTune, and Xeon are trademarks of Intel Corporation or its subsidiaries.

About Acer

Founded in 1976, Acer is one of the world's top ICT companies with a presence in more than 160 countries. As Acer evolves with the industry and changing lifestyles, it is focused on enabling a world where hardware, software and services will fuse with one another, creating ecosystems and opening up new possibilities for consumers and businesses alike. Acer's 7,500 employees are dedicated to the research, design, marketing, sale, and support of products and solutions that break barriers between people and technology. Please visit www.acer.com for more information.